FIDO/FIDONET ROUTING, TOPOLOGY, HISTORY AND RECENT CHANGES

Tom Jennings, 1:125/111

Fido/FidoNet, like all other FidoNet mailers and BBSs, generates

messages, and puts them into packets that are later delivered to

some appropriate destination by the mailer itself. All of the

different mailers use different approaches as to just how you the

sysop control where, how and when packets (and the messages they

contain) get delivered.

In light of all the mailer systems out there today, I don't think

many are aware of just how Fido/FidoNet does it's routing. With a

few recent changes (more to follow) you might find the design has

become interesting.

FIDO

Fido was originally just a bulletin board; the first FidoNet was

a separate program that was run from a batch file with a few

small hooks into the BBS. (The origin of the Fido version 9 - 11

MAIL.SYS file.) Fido (the BBS) only let users generate messages;

FidoNet (the mailer) put messages into packets and delivered

them.

At this point, five years later, Fido and FidoNet are pretty well

integrated, and this latest revision completes the weld.

Logically, to the user and sysop, the two remain quite separate,

and many (non-FidoNet) Fido systems are BBS only. (Most of my

commercial customers are BBS only.) It is just as easy to run

FidoNet without Fido.

Fido's packeting/mailing system works in four discrete phases.

First, the destination node addresses for all the existing

messages is determined. This is done by the "router", more on

which follows. Second, the messages are put into packets by the

"packeter" (I never was very good at names). Third, the phase

that is most obvious to sysops watching the screen, is when the

packets are delivered; Fido makes outgoing phone calls and sends

the packets. Packets can also be received inbetween outgoing

calls. The last phase deletes un-sent packets, and marks the

original messages that went into the packets as "(SENT)" as

appropriate. This ends the FidoNet session.

Note that unlike Opus and other similar mailers, Fido only puts a

copy of the message into a packet; during the fourth phase Fido

again processes each message, and marks it or deletes it as

determined by the success of that packet delivery.

This is a fairly large amount of processing to do when looked at

on a per-message basis, and is why Fido's FidoNet has always been

slower to packet than other systems. In return there are many

advantages, that will become more obvious later.

FIDO AND FIDONET

Originally, as was stated before, Fido and FidoNet were two

separate programs. Even when integrated into one package,

starting with Fido version 9 or 10, FidoNet was only usable when

a FidoNet scheduled event was actually running; "continuous mail"

is (relative to Fido) a new concept. Version 12 (Aug. 1987) could

accept incoming continuous mail, but not send mail unless a

FidoNet event was running. Version 12M supports Wazoo and file

requests.

Starting with version 12N, the FidoNet portion of Fido can be

accessed at any time; packet creation and routing is under

complete control, and can be altered, automatically using the

routing language on a event by event basis throughout the day, or

manually as the sysop sees fit, up to the point when the specific

message has been delivered. Events themselves can be turned on

and off from within Fido, allowing very high-level control over

packet routing.

You can have Fido create packets available for pickup, with any

arbitrary routing, at any time of day. For example, you can have

HOLD packets of long-distance systems waiting for pickup from

9:00AM til 6:00PM, while enabling outgoing calls on local-dial

systems, in between human callers, or any other construct allowed

by the routing language, without restriction. There is a

"penalty" of 30 - 60 seconds to prepare for a new schedule; once

started, access is in the under 100 mS range.

On my 8MHz "turbo" junk-pclone, 80mS 20 meg drive, Fido takes 20

seconds to load, create outgoing packets and be ready for an

incoming call (human or otherwise). On this crappy hardware,

incoming echomail is received, unpacketed, tossed, the echo areas

then scanned and outgoing packets made and delivered in 60 - 90

seconds, in between human callers, using DCM and barefoot

Fido/FidoNet 12N.

The largest network Fido/FidoNet can (mathematically!) handle is

(32767 * 32767 * 32767) or 3.5 x 10(13) nodes; version 12's

implementation 65,000. A recompile (change a table index from 16

to 32 bits) will make Fido handle about 4 billion nodes with some

performance loss and increased (disk) overhead, about 2

bytes/node. Performance with 65,000 nodes would still be better

than Fido 12M's.

Current nodelist overhead (NODELIST.132) is: NODELIST.BBS 304,532

(physical data); NODELIST.NMP 53,920 (nodemap; see below);

NODELIST.IDX 53920 (main index); NODELIST.NDX 2900 (host index).

NODELIST.SYS is no longer used.

FIDONET TOPOLOGY

The router design mimics exactly the FidoNet network topology.

The network went through three (so far...) stages: a "flat"

system, ie. point to point; addresses were a simple number 1 -

32767. The second formalized the concept of "nets", incorporating

the routing optimization formerly done with Fido's primitive

router. Third, the current scheme, includes zones, which are

similar mathematically to nets, but in real life act quite

differently, with "zone gates" concentrating mail between zones

(generally continents) because of real-life issues of telephone

connect costs and equipment compatibility.

OOPS BACKTRACK A LITTLE:

A small aside on nets and regions: "regions" originally were only

a way for nodes not in a net (ie. not inside a local calling

area) to be syntactically compatible with the "net/node"

addressing scheme; since most nodes were in the most heavily

populated areas, cities, where nets naturally form, "regions"

would be where nodes not in cities would be found. Nodes in

regions (marked REGION in the nodelist) act as any other node,

but the mailers do not do the automatic routing to the "host" for

the region -- mail is sent direct, or point to point.

Additionally, the function of region hosts as another layer of

organizational heirarchy is a recent addition, and not part of

the topology itself. Still further, there is nothing magic about

the numbers themselves -- regions being numbered 1 - 99, nets 100

- 999 etc is a totally arbitrary decision on the part of the

keepers-of-the-lists. The only magic numbers are 0's -- these

indicate the host for the entity, ie. zone, net or region.

ROUTER DESIGN

Back to the router design. While the heirarchical model of

net/node is extremely useful (if not indispensible) there are

still thousands of exceptions, usually on a system by system

basis; you forward mail for one system that is local but is a

toll call for other net members. Your net has a sugar daddy that

can make long distance outgoing calls. One system calls in to

pickup their mail. Commonly called systems are more efficiently

handled in some special way.

Fido's router design can handle any topology based on our address

syntax: zone:net/node, plus any arbitrary number of exceptions.

To do this, the router is very simple -- not complex.

Logically, the router is an N x N crossbar switch, where N is the

number of nodes in the nodelist. You can imagine a crossbar

switch by drawing on paper a grid:

IN

--> 1 ----O---O---O---O---O

|   |   |   |   |

2	----O---O---O---O---O

|   |   |   |   |

3	----O---X---O---O---O

|   |   |   |   |

4	----O---O---O---O---O

|   |   |   |   |

5	----O---O---O---O---O

|   |   |   |   |

1   2   3   4   5

OUT

Shown is a 5 x 5 crossbar switch. The O's represent an OFF (but

potential) connection; X's represent a ON connection. The

connection (3,2) is ON, all others closed. If a signal were

applied to Input 3, it would appear also on Output 2. (ASCII

graphics are terrible, sorry!) You will notice that by placing

X's and O's appropriately, any input can be connected to any

output.

A "real" crossbar switch can route one signal to many

destinations; just place X's along the same horizontal row in the

example above. Any node can route to any node; times (N) nodes is

(N * N) possible states. Not pleasant to think about in real

terms -- a 5000 node nodelist would mean 25,000,000 states to

represent on your disk! This is not a very useful side effect for

us; our messages have a single destination address.

Fido's router places one limitation upon the crossbar design:

there can be only one possible destination per node. It can still

be any possible node, but only one at a time. This means the

router can consist of (2 * N) entries -- the originating node and

the destination node.

You can imagine Fido's router as the crossbar switch above, or as

I do, a simple two column table:

----+----

1   |	_

2   |	_

3   |	2

4   |	_

5   |	_

The _'s represent potential, but OFF connections. #3 has been

routed to #2 by merely filling in that table entry. This table is

called the NodeMap.

(Fido's nodemap also contains a third column, where attributes

like HOLD, SEND-TO, PICKUP and other things are stored. These

atributes are built into the nodemap for programming convenience

only, they are not really part of the router per se.)

HOW THE ROUTER WORKS

At FidoNet mail time, Fido prepares the router files before

making packets and outgoing phone calls. The basic net host

routing is performed, then any routing specified by the sysop in

route language files.

Before any routing, the table looks like this:

ADDRESS		ROUTE-TO	ATTRIBUTES

1:1/1		1:1/1		  (none)

1:1/2		1:1/2		  ...

...		...		  ...

1:125/0		1:125/0

1:125/20	1:125/20

1:125/111	1:125/111

...		...

2:500/0		2:500/0

2:500/2		2:500/2

...		...		  ...

Basic default routing is applied, which does the FidoNet-as-we-

know-it net and zonegate routing (see the Appendix A: DEFAULT

ROUTING section):

ADDRESS		ROUTE-TO	ATTRIBUTES

1:1/1		1:1/1		  ...

1:1/2		1:1/2

...		...

1:125/0		1:125/0

1:125/20	1:125/0

1:125/111	1:125/0

...		...

2:500/0		1:1/2

2:500/2		1:1/2

...		...

At this point Fido performs any additional routing you may have

specified, such as overriding the routing, HOLD packets, enabling

only certain nodes or groups of nodes per schedule, etc. Things

like HOLD, PICKUP, SEND-TO and other basic concepts are as

attributes within the nodemap.

The nodemap is built on disk, and can be saved between schedules

so that it an be used over and over; this is called a "QUICK"

FidoNet event. It takes my Fido system mentioned above

approximately 90 seconds to completely build the nodemap (about

100 route language statements); subsequent "QUICK" events take a

fraction of a second.

PACKET CREATION

Fido creates packets when a FidoNet schedule starts (which is

controlled by Fido's scheduler and is outside this discussion).

For every message in the netmail message area, Fido consults the

nodemap, in two steps:

First, the actual destination (for example: 1:125/111) is looked

up in the ADDRESS column of the nodemap. The ROUTE-TO column

determines where this message goes, ie. into which packet. If the

destination node is not found, the message is marked (ORPHAN).

Secondly, Fido looks up the packet (ROUTE-TO) address (1:125/0)

itself, in the ADDRESS column. This is done to locate the

ATTRIBUTE bits for the destination node. If the bits indicate it

is OK to packet this message (SEND-TO set, etc) then the packeter

creates the packet.

This is done for all messages in the netmail area; once all the

packets are built then FidoNet can dial out, allow incoming

pickups, etc.

Messages put into packets are not modified in any way; packets

contain a copy of the original message. The post-FidoNet process

takes care of messages that have been sent.

FIDONET SESSION COMPLETION

When a FidoNet schedule is over, Fido processes packets that were

received from other mailers and cleans up any packets it had

created earlier.

Packets that are un-sent are merely killed; the messages that

these packet(s) were created from still exist in the netmail

area; when a FidoNet session start again, Fido may put the

messages into a packet to the same destination node or possibly

another; since packeting is done only before actual mailing the

routing can be altered at any point up to actual sucessful

transmission.

Packets that are sent, or picked up, are handled slightly

differently. The packets themselves are deleted, but Fido once

again refers to the router to mark the messages that comprised

the packet as (SENT), or kills them if they were indicated

(KILL/SENT) by the originator.

Appendix A: DEFAULT ROUTING

Fido/FidoNet's routing is not "built-in" nor hard-coded; if it

were not told otherwise, Fido would send messages to the

destinations in the message itself. The routing needed to make a

practical mailer are added as layers upon this base; the tradeoff

is speed vs. flexibility and accuracy. (Speed is, um, somewhat

improved over older implementations...)

What the real-life Fido does at FidoNet mail time is make a pass

through the table, and fill in the "default" routing that defines

the FidoNet tolopogy, which is our zone:net/node with routing to

HOSTs for nets, which goes like this:

-For nodes in our own net, send direct (point to

point)

-For nodes in a net in our zone, outside our net,

send to it's host (net/0)

-For nodes in a region in our zone, sent direct

-For nodes in another zone, send to it's zone

host (zone:0/0)

The first three make sense in the network as we know it; the

fourth requires some background.

FidoNet's topology is based upon a gimmick: the address of the

logical host for any net or zone is composed ot the number of the

net or zone, with the magic zero added as the least signifigant

address field. A net or region host is net/0 or region/0; a zone

host is zone:0/0. FidoNet sysops use net/0 routinely; no one uses

zone:0/0 routinely, if at all.

The difference is that the addressing scheme, the topology, is a

mathematical construct, and has nothing to do with the real

world, ie. overseas phone calls, governmental regulations,

manufacturer incompatibilies, etc. The addressing scheme needs to

be rigorous and provide a solid design base for all

implementations.

If we didn't have real-life complications like the above, never

mind how overloaded the poor zone host computer would be, the

mathematical model might fit the real world. Obviously it

doesn't, and never did.

The solution in Fido's scheme is to merely modify the default

routing. There exists a keyword in Fido's routing language

(called, not surprisingly, "ZoneGate") that does exactly what it

sounds like: it routes all mail destined for another zone to any

arbitrary node designated "zone gate".

Zone Gates were thunk up at the now notorious "New Hampshire

meeting" in '86 or so. The idea was to make it so that net/node

mailers, ie. not zone-aware, could route messages destined for

other zones. The thing was called the "IFNA Kludge", and consists

of two parts: (1) an addressing kludge to trick the mailer to

route the interzone message to a node in it's own zone, and (2)

to have the full zone:net/node origination and destination

addresses buried in the message body itself, hidden behind a line

that began with Control-A, so that message editors could learn to

ignore it. (For your curiosity: full address consists of the very

first line in the message, that looks like: "^AINTL z:n/f z:n/f",

where the first address is the destination node address, the

second the originator.)

The addressing trick is: "Address the message for zone (N) to

node 1/(N) in my zone". Node 1/(N) is designated the zone gate;

for example, the zonegate for Europe, Zone 2, node 1/2, in the

North American zone 1. And so on.

Fido is a registered trademark of Tom Jennings

FidoNet is a registered trademark of Tom Jennings

(Sorry, I gotta say this!)