Fidonet Electronic Mail Protocol 20 Oct 84        Page 1

Tom Jennings

2269 Market Street #118

San Francisco CA 94114

FidoNet Node #1

415/864-1418

INTENT

This document is an attempt to describe and define

the packet protocol used to support electronic mail under

the Fido/FidoNet system.

It will not describe or define Fido/Fidonet software

operation, nor even describe what the system does; for that,

I refer you to the following Fido manuals:

Fido's Installation Manual      (INSTALL.DOC)

Fido's Operating Manual         (FIDO.DOC)

Fido's FidoNet Manual           (FIDONET.DOC)

Only the lower layers of the protocol will be

discussed here; routing, forwarding, dialing algorithms and

such will not be covered. These are mentioned, albeit

briefly, in Fido's FidoNet Manual.

BACKGROUND

FidoNet was designed to be more or less portable

within the existing "public domain" software base, ie. any

system that can support binary transfers via XMODEM or it's

variants is in theory capable of supporting FidoNet. This

does not mean to imply that the current Fido/FidoNet

software will be/can be transported to machines not

currently supported; it means that no designed in

limitations were intended.

This document merely covers the algorithms used, and

the problems encountered, in the automatic transfer of

packets and files used by the MSDOS Fido/FidoNet system.

DESCRIPTION

FidoNet is a true electronic mail system supported

by the Fido Bulletin Board System software, under MSDOS

version 2. It's function is to transfer textual messages,

and binary files, between physically seperate computers on

an automatic, unattended basis.

BASIC ASSUMPTIONS

The basic unit of data in FidoNet, like most MSDOS

and CPM software, is the byte, which here means 8 bits. All

data transfers assume the availability of an 8 bit byte

capable channel; 7 bit ASCII is not adequate. Parity, if

Fidonet Electronic Mail Protocol 20 Oct 84        Page 2

used, must be transparent to the software.

FidoNet is transaction oriented; a channel

(connection) is made, data transferred, the channel closed.

In all existing cases, this means a typical modem/phone line

connection; dial, connect, transfer, disconnect, though

anything that might be considered a "channel" will work.

The most basic design criteria for FidoNet is

extremely short channel connect times, since the usual

channel is a telephone line, where costs are accumulated on

a per minute basis. Lowest possible costs is the single most

important issue here. For this reason, all packets are

preassembled; no processing at all is done at packet

transfer time.

Due to the lack of real time operating systems,

FidoNet runs on an alternating basis with Fido. While Fido

is running, users enter messages, attach files and the like,

to be later processed by FidoNet. When FidoNet gains control

(at a predetermined time of day) it packetizes outgoing

messages, then makes attempts to send these packets, and may

receive packets from other nodes, all asynchronouously. When

FidoNet's time slot is over, it deletes any un-sent packets,

unpacks any packets that were received, and then passes

control back to Fido. Note that incoming packets are not

examined in any way until the FidoNet time slot is over.

A basic concept in FidoNet is the "node". A node is

any computer capable of sending or receiving FidoNet mail

packets. All discussions of packet transfers here will

assume point to point transfers.

As background information, it should be noted that

routing, forwarding, cost accounting, etc is done at a much

higher layer; routing, for instance is done even before

packets are created, and determines the destination node. It

has no effect on packet transfer.

SOFTWARE ASSUMPTIONS

Fido/FidoNet is written in 'C' (Lattice flavor) and

makes use (of course) of C style structures and strings.

Luckily, these are quite simple. Here are the following

basic components used throughout FidoNet:

CHARACTERS:

An 8 bit byte. The usual micro type "character"; all

8 bits may be signifigant. Usually 128ASCII.

STRING:

A one dimension array of 8 bit characters,

terminated by a null. (hex 0) Some are of a fixed length; in

this case, signifigant text is left justified, followed by

the null, and the rest of the string is indeterminate. In

practice, FidoNet strings vary from very few characters up

to 4000 characters or so.

Fidonet Electronic Mail Protocol 20 Oct 84        Page 3

INTEGERS:

All integer variables are 16 bits ('C' shorts)

unsigned, and are in Intel (big endian) format; least byte

first in the byte stream.

LONGS:

A long integer is 32 bits, in Intel format.

WHAT A PACKET IS

A packet is a file that consists of a header, a

variable amount of variable size messages, and a trailer.

Each message within a packet has a header, and a single

string of plain text.

+---------------+

|               |

|               | Header

|               |

+---------------+

|               | Message #1

+---------------+

|               |

/               /

/               /

|               |

+---------------+

|               | Message #N

+---------------+

|               | Trailer

+---------------+

The header and trailer are of fixed length. All of

the messages are variable length, but have a fixed number of

fields within them.

Packets must be treated as a byte stream; for many

of the components, "type" flags determine the format of

upcoming data, therefore it is difficult to "read ahead",

except for buffering purposes.

THE HEADER:

The header is a fixed length structure, and contains

the originating and destination nodes, the date the packet

was made, a version number, and some extra space for future

expansion.

Foolishly, there is no "type" identifier in the

packet header.

A PACKET MESSAGE:

Each message contains an interger "type", followed

by the message fields. These are: attribute, originating

Fidonet Electronic Mail Protocol 20 Oct 84        Page 4

node, destination node, two zeros, cost, date string, "to"

string, "from" string, "Subject" string, followed by the

text string.

The type, attribute, orig and dest node numbers, the

two zeros and cost are all integers. The other are serial

byte streams, strings, of variable length.

INPORTANT NOTE:

Due to extreme shortsightedness in the original

packet design, the message type 1 did not include the

originating and destination node numbers; FidoNet inserted

the node numbers found in the packet header when the message

was unpacked and stored in the system.

Routing meant that messages for many different nodes

were packed into a single packet; needless to say this means

that the node numbers may not be the same for each message,

as was the previous assumtion.

A message type 2 (the one described above) was added

to accomodate the "extra" information. Starting in version

10, FidoNet will no longer create message type 1, though it

will continue to receive them. Other implementations may

ignore message type 1, and assume that all are message type

2.

THE TRAILER:

The trailer consists of a single integer 0. This can

be thought of as a message type 0. This ends the packet; any

data following it (as when XMODEM rounds the packet up to

the next highest 128 bytes) should be ignored.

SYSTEM ASSUMPTIONS

Fido and FidoNet were designed using some basic

software building block concepts, admittedly stolen from

Ward Christensen's MODEM program.

XMODEM with CRC error checking is used for the basic

mail packet transfer. No further discussion will be made as

to XMODEM implementation nor operation. Please refer to

other documents.

MODEM7, or more accurately, TELINK, protocol is used

to transfer any files "attached" after the message packet.

TELINK is identical to MODEM7, with the addition of an extra

data block, that passes MSDOS dependent file information

(time, date, exact file size) before the first data block.

FidoNet is still MODEM7 capable; all that is necessary is to

NAK the extra block, whose header character is ASCII SYN

instead of SOH when receiving from a TELINK based FidoNet.

For sending to TELINK based receiving FidoNets it is not

necessary to send this extra block. This protocol is also

Fidonet Electronic Mail Protocol 20 Oct 84        Page 5

described elsewhere.

FidoNet assumes the presence of a clock of some

sort, for failsafe monitoring of packet transfers, and for

accounting purposes, to monitor actual connect times.

Accounting is outside the scope of this document.

A clock timer, preferably interrupt driven, is

needed to monitor the initial connection process, though may

be done using polled counters, if they are "calibrated".

This is fairly important; it prevents excessive connect

times if either the sender or receiver fails at any point.

There are two basic subroutines used in the Fido

implementation of FidoNet. While these are very low level

hardware dependent routines, they are very important in

concept.

DELAY ( N )

Delay for N centiseconds. This is just a time delay,

or time waster routine. It is used for generating time

delays for accomodating the telephone companies equipment.

MODOUT ( CHARACTER )

Send a character to the modem. This is shown for

illustration purposes; it is used in the diagrams to follow.

MODIN ( N )

Receive a character from the mode, but wait no

longer than N centiseconds. If a character is received, it

is returned; if it times out, -1 is returned. Since

characters are 8 bits, -1 (16 bits) indicates that no valid

character was received. This is a very basic, very important

function, upon which all of FidoNet is based.

The centisecond timer does not need to be accurate.

10% or less error is ideal, 20% is acceptable, and Fidonet

will usually still operate at up to 100% error. However,

repeatability will suffer, and sensitivity to the "length"

of a telephone connection will increase rapidly above 25%

error.

This is implemented on MSDOS as an interrupt driver

timer; a polled timing loop is adequate if adjusted to match

the processor speed.

Fidonet Electronic Mail Protocol 20 Oct 84        Page 6

PACKET TRANSFERS

Packet transfers will be described in two ways here;

first, a prose description, lacking exact details, and a

diagram with tables, with exact timing relationships and

values.

A FidoNet session consists of the following:

- Building routing tables

- Building list of active nodes

- Creating packets

- Compiling Messages

- Compiling list of attached files

- Sending/receiving Packets

- Deleteing outgoing packets, marking sent messages

- Unpacking incoming packets

- Return control to Fido

Only the creating of packets and attached file

lists, their transmission and reception, and unpacking is

covered here. The other steps are Fido dependent, and will

vary from implementation to implementation.

CREATING PACKETS:

FidoNet makes a number of passes through all of the

messages in the system. The first pass is merely to

determine which nodes have messages destined to them. Each

node is marked in a table, for later use in making packets

and dialing.

Additional passes are made, one for each node there

is mail to. A packet file is created on disk, using the

destination node number to create a unique filename (44.out

for a packet to node 44, etc), and the packet header is

written out.

When a message to this node is found, the message is

written out to the packet file. (The messages as stored in

Fido are described later in detail)

After the last message is written to the packet, the

trailer is written out, the file closed.

This process is repeated for all nodes that have

mail to be sent.

Fidonet Electronic Mail Protocol 20 Oct 84        Page 7

SENDING PACKETS:

RECEIVING PACKETS:

FidoNet alternates sending and receiving packets for

most of the time slot. When it is not actually sending (or

attempting to send) a packet, it is waiting for one.

If there are no incoming calls, FidoNet makes

outgoing calls at a random one or two minute interval. A

node is chosen to call from the table compiled while making

packets. A node is picked sequentially from this table,

trying each one in sucession. When the end of the table is

reached, it starts over at the beginning.

When it is time to place a call, the next node is

chosen from the table, the telephone number is dialed. If no

connection is made, FidoNet goes back into receive mode.

If a connection is made, CRs are sent, to determine

the baud rate. When the called system determines the baud

rate, it sends the "signon" message telling human callers it

is not available. This message is detected by the caller,

and terminates the baud detection. If baud is not detected

in 8 seconds, the caller disconnects.

The sender then waits for the line to clear (the

signon message to stop), then sends a character designated

as "TSYNC". This tells the receiver that it is a FidoNet

node calling, instead of a human caller. There is a 60

second limit placed on the TSYNC sequence. The receiver then

goes into XMODEM file receive, with a unique name for the

incoming packet. (0.in, 1.in ...)

After sending TSYNC, the sender transmits the packet

(N.OUT) using normal XMODEM protocol.

After the packet is sent, the receiver goes

immediately into MODEM7 mode, to receive any attached files.

If there are no attached files, the sender sends an

EOT, which when received by the receiver in MODEM7, tells it

"no more files". If there are attached files, the sender

sends all of the files in the attached file list, using

MODEM7.

After either sending the attached files, or after

sending the EOT meaning no attached files, the sender delays

five seconds, then disconnects. After receiving the last

attached file (or the EOT) the receiver delays two seconds

and disconnects. The delay is to accomodate the telephone

systems peculiarity of keeping connect information on a

seperate, faster, channel that the data itself; without the

delay, it is possible for the "disconnect" to get there

before the character does.

If the packet was sent sucessfully, the sender

deletes the outgoing packet, the attached file list, and

Fidonet Electronic Mail Protocol 20 Oct 84        Page 8

marks the node table as "sucessful", and then returns to the

receive and wait mode.

UNPACKING PACKETS:

When FidoNet mail time is over, any unsent outgoing

packets are deleted, and all messages that were sent

sucessfully are marked as SENT.

Each incoming packet (0.in, 1.in ...) is unpacked

and deleted. First, the destination node number is compared

to the nodes actual number. If it does not match (this

packet not really meant for this node) the packet is deleted

without further unpacking. If it matches, the messages are

unpacked and placed in the message area.

The packet is read as a byte stream, and interpreted

as read. When a message type is found, FidoNet assembles the

message as it goes.

Note that it is possible to receive part of a

packet; Fidonet will unpack all complete messages and store

them in the message area. It may find an error, such as an

incomplete packet. This is not a necessary feature; since

the packet was aborted, it will be sent again the next

night.

Fidonet Electronic Mail Protocol 20 Oct 84        Page 9

FIDONET DIALING AND PACKET TRANSFER

This is the final, detailed, description of the

actual packet transfer. Some shorthand notation will be used

here; "clock" means a general purpose timer for monitoring

elapsed time, and is set and read during many parts of the

packet protocol. The two subroutines described above are

also used through out.

The descriptions her are in a C like "pseudo code",

for simplicity. Function or subroutine calls are shown as:

function(parameter);

Where function() is the subroutine, and "parameter"

is a parameter passed to the function. Parameters here may

be numeric values or strings, which can be determined from

context. Functions without parameters are shown as

function(). Functions may return a value, so that:

x= function()

Sets variable X to the value returned by function().

IMPORTANT NOTE:

Ther is one extremely important construct that does

not, and cannot show up in these diagrams. This is a

mechanism to monitor Carrier Detect on the modem, and return

directly to a high level subroutine for error handling. This

is important due to the huge number of place where carrier

loss would have to be checked for, with the corresponding

high number of error return checks.

This can be implemented with a C like "set_jmp",

which stores the stack pointer and a return address, and

jumps to the return address if carrier is lost.

Also, in the Fido implementation, this error exit is

taken when the clock goes over the set limit. This frees the

high level code from having to watch the clock constantly.

The actual high level packet receive and send C

source is included at the end of this document to hopefully

make this a bit clearer.

All mnemonics are standard ASCII control characters,

with the exception of "TSYNC", described below.

Control Character Definitions

Char    Hex     Dec

----    ---     ---

CR      0d      13

EOT     04       4

TSYNC   ae     174

Fidonet Electronic Mail Protocol 20 Oct 84        Page 10

PACKET TRANSMISSION:

send_packet:

Attempt to place the call. If no connection, just return,

where FidoNet will wait for an incoming call, or delay one

or two minutes to place the next outgoing call.

set baud rate from node list

dial phone number,

if no connection, return

A connection has been made. Determine the baud rate first.

This "whacks return" up to eight times; if no message

appears from whacking, it stops and calls it an error. The

"double whack" (sic) seems to work better than a single CR.

When the receiver determines the baud rate, it sends 4 CRs,

and the signon message, which also contains CRs. The sender

notices this and stops sending its CRs.

for (i= 1 to 8) {

send CR, delay(20), send CR;

if (modin(100) == CR) break out of loop;

}

if (no CR received above) {

mark table as one "connect",

disconnect,

return.

}

Baud rate is now detected. Set the failsafe timer, wait

until the line clears (the signon message stops coming),

then send TSYNC. If one minute elapses, disconnect and

return.

print ("Connected");

set clock to zero;

while (clock is less than one minute) {

if (modin(50) == -1) break out of loop;

}

if clock == one minute {

disconnect,

return.

}

send (TSYNC);

disable clock;

print ("Sent TSYNC");

Presumably, all connected now. Start sending the packet via

XMODEM. XMODEM will timeout after so many tries, so that

this will not hang if the other end fails or is not a

FidoNet.

Fidonet Electronic Mail Protocol 20 Oct 84        Page 11

print "Sending Packet"

xmodem send (packet);

Packet transmission complete. Mark the in memory table as

"succesful". If there are attached files, send them now,

otherwise just send an EOT.

success= 1;

if (attached files)

modem7 send attached files

else send EOT.

All sent, delay 5 seconds to let things settle down.

delay(500)

disconnect,

return.

Fidonet Electronic Mail Protocol 20 Oct 84        Page 12

PACKET RECEPTION:

Carrier was detected while waiting to place a call or

waiting for an incoming call. Determine baud rate first,

display the signon message, then wait for TSYNC. There is a

one minute limit waiting for TSYNC. Note that connect()

watches the clock while waiting for CRs, and will take a

fatal error abort if it goes over the set limit. This

example therefore assumes that it will not timeout.

receive_packet:

reset clock,

connect()                       determine baud rate

print to modem:

"FidoNet Version x.x

FidoNet Node #N

Processing mail Only"

while (clock less than one minute) {

if modin(100) == TSYNC break out of loop;

}

if (over one minute) {

disconnect,

return;

}

All connected, received TSYNC. Wait for the incoming packet.

Each incoming packet is named sequentially, starting at 0;

0.in, 1.in, 2.in, etc. If no packet was received, quite now;

no sense in trying to get attached files if no packet

received.

xmodem receive packet

if no files received {

disconnect,

return.

}

Since it may have taken a few seconds to close up and write

out the packet file, the sender may have sent extra EOTs

from the end of the XMODEM packet transfer. Delay a bit, and

flush out the modem.

delay(200),

flush modem,

Now just go right into MODEM7 receive, for any attached

files. If there are none, the sender will send us an EOT,

which tells MODEM7 there are no more files. It can also just

hang up at this point, since the packet has already been

received. After the last, if any, file(s) received, delay 2

seconds and disconnect.

receive modem7 *.*

Fidonet Electronic Mail Protocol 20 Oct 84        Page 13

delay(200),

disconnect,

return.

Fidonet Electronic Mail Protocol 20 Oct 84        Page 14

IMPLEMENTATION NOTES:

These are some suggestions on implementing FidoNet

on all systems. They are merely suggestions; obviously you

can do what you want.

LOW LEVEL FUNCTIONS:

As mentioned before, there are some low level

functions that at least some of which are probably

universal. The ideas below may make it easier to implement a

"bullet proof" FidoNet.

ELAPSED TIME CLOCK:

This is a necessity, in one form or another. It's

accuracy does not need to be high. In Fido, it is

implemented as a timer tick interrupt, that increments in

memory variables: 'millisecs', 'seconds', 'minutes', and a

function to clear the clock, ie. set the above counters to

zero. (millisec is almost never actuall milliseconds; on the

IBM PC, for instance, the clock tick is at 55.55 mS; 55 is

added to 'millisec' every clock tick.)

There is an additional variable called 'limit', that

is the time limit to enforce for a particular routine.

Setting it to zero effectively disables the limit.

Thirdly, there is a function called limitchk(), that

does two things:

1.      Watches the set time limit vs. the minute counter

2.      Watches the carrier detect line from the modem

If either one of these events happens, it takes the

fatal error trap, and returns directly, instead of returning

normally.

limitchk() {

if (limit > 0) {

if (minutes >= limit) frc_abort();

}

if ! dsr()) frc_abort();

}

frc_abort() is the fatal error jump, back to the

caller directly. dsr() is a function that returns false (0)

if carrier is lost. If no fatal errors, limitchk() just

returns.

MODIN(N)

MODOUT()

Fidonet Electronic Mail Protocol 20 Oct 84        Page 15

These two are the lowest level character IO in Fido.

modin(N) returns either a character from the modem, or -1

(TIMEOUT) if no character was available within N

centiseconds. This avoids hanging forever waiting for a

character that may never come.

modout() merely sends a character to the modem.

Both of these functions check for errors by calling

limitchk() while waiting for input or output ready.

modin(n)

int n;

{

millisec= 0L;

while (millisec < (10 * n) ) {

if (character ready) return(modem char);

limitchk();

}

return(-1);

}

modout(c)

char c;

{

while (output not ready) {

limitchk();

}

send char to modem

}

Fidonet Electronic Mail Protocol 20 Oct 84        Page 16

MESSAGES

While the message format is very Fido dependent, it

may be illustrative to show the correspondence between a

"normal" Fido message and one that has been installed in a

packet.

This is the format of a Fido message, as contained

in any Fido Message Area.

/* Message header structure. The message text is just a long

string. */

struct _msg {

char from[36];          /* who from, */

char to[36];            /* who to, */

char subj[72];          /* message subject, */

char date[20];          /* creation date, */

int times;              /* number of times read, */

int dest;               /* destination node, */

int orig;               /* originating node */

int cost;               /* actual cost of msg */

int caca[6];            /* extra space, */

unsigned reply;         /* thread to previous */

int attr;               /* message type, */

int up;                 /* thread to next */

};

/* The message text starts at the byte following the end

of the structure, and is terminated by a single null. */

/* Bit values for msg.attr */

#define MSGPRIVATE 1            /* private message, */

#define MSGREAD 4               /* read by addressee */

#define MSGSENT 8               /* sent OK (remote) */

#define MSGFILE 16              /* file attached to msg */

#define MSGFWD 32               /* being forwarded */

#define MSGORPHAN 64            /* unknown dest node */

#define MSGKILL 128             /* kill after mailing */

A Fido message is 191 bytes minimum; 190 bytes of

header, above, and a variable amount of message text, null

terminated. A fairly large klunky thing to pass around. When

compacted into a packet, it is much smaller. As an example,

assume the following message:

From: Tom Jennings  On Fido 1

To: John Madill  On Fido 2

Subject: Test message

This is text in the test message.

A hex dump of this test message follows. Note that

the null terminated strings are followed by garbage; please

ignore it.

Fidonet Electronic Mail Protocol 20 Oct 84        Page 17

54 6F 6D 20 4A 65 6E 6E-69 6E 67 73 00 00 8E 36   Tom Jennings...6

00 00 00 FE FF 06 00 00-00 00 96 36 31 00 F3 21   ...~.......61.s!

0A 00 00 00 4A 6F 68 6E-20 4D 61 64 69 6C 6C 00   ....John Madill.

86 99 F8 27 86 99 02 28-00 00 91 36 57 61 69 74   ..x'...(...6Wait

20 2E A0 99 AE 16 68 21-74 65 73 74 20 6D 65 73    . ...h!test mes

73 61 67 65 00 00 65 00-FF 00 FC FF 00 00 B8 99   sage..e...|...8.

40 00 AF 06 AF 06 65 06-40 00 65 00 65 00 FC FF   @././.e.@.e.e.|.

00 00 AF 06 0D 06 40 00-65 00 FF 00 FC FF 00 00   ../...@.e...|...

DA 99 65 00 AF 06 7E 60-1D 1A 83 F2 99 36 BC 27   Z.e./.~`...r.6<'

32 33 20 4F 63 74 20 38-34 20 20 31 36 3A 31 39   23 Oct 84  16:19

3A 34 38 00 01 00 02 00-01 00 1B 00 00 00 00 00   :48.............

00 00 00 00 00 00 00 00-00 00 81 00 00 00 20 20   ..............

54 68 69 73 20 69 73 20-74 65 78 74 20 69 6E 20   This is text in

74 68 65 20 74 65 73 74-20 6D 65 73 73 61 67 65   the test message

2E 0D 0A 0D 0A 00                                 ......

When the message is packetized, it becomes much

smaller; an entire packet, header, the above message and

trailer, is only 164 bytes. The extra space and most of the

Fido dependent items are removed.

01 00 02 00 C0 07 09 00-02 00 10 00 19 00 0E 00   ....@...........

00 00 01 00 00 00 00 00-00 00 00 00 00 00 00 00   ................

00 00 00 00 00 00 00 00-00 00 00 00 00 00 00 00   ................

00 00 00 00 00 00 00 00-00 00 01 00 01 00 1B 00   ................

32 33 20 4F 63 74 20 38-34 20 20 31 36 3A 31 39   23 Oct 84  16:19

3A 34 38 00 4A 6F 68 6E-20 4D 61 64 69 6C 6C 00   :48.John Madill.

54 6F 6D 20 4A 65 6E 6E-69 6E 67 73 00 74 65 73   Tom Jennings.tes

74 20 6D 65 73 73 61 67-65 00 20 20 54 68 69 73   t message.  This

20 69 73 20 74 65 78 74-20 69 6E 20 74 68 65 20    is text in the

74 65 73 74 20 6D 65 73-73 61 67 65 2E 0D 0A 0D   test message....

0A 00 00 00                                       ....

The packet header format is as follows. It is

possible to follow, and hand disassemble, the message

packet.

struct _hdr {

int orig;               /* originating Node # */

int dest;               /* destination node */

int year,month,day,hour,minute,second;

int rate;               /* baud rate */

int ver;                /* packet version */

int extra[19];

} pkthdr;

Fidonet Electronic Mail Protocol 20 Oct 84        Page 18

#include <stdio.h>

#include <ctype.h>

#include <bbs.h>

#include <xfbuf.h>

#include <ascii.h>

#include <xtelink.h>

#define TSYNC 0xae

extern char ver[];

extern unsigned crcmode;                /* 1 if CRC mode, */

extern unsigned filemode;               /* transfer type; XMODEM, MODEM7, TELINK */

/* Statistics on file transmission. */

extern unsigned totl_files;             /* total files sent/rec'd */

extern unsigned totl_failures;          /* how many failed, */

extern unsigned totl_errors;            /* error count, soft errors incl */

extern unsigned totl_blocks;            /* number blocks sent, */

extern int minutes,seconds;

extern int pktnum;                      /* incoming packet number */

extern struct _route route[ROUTE_SIZE]; /* routing table */

extern int maxnode;                     /* number of nodes in route table */

extern struct _sched sched[SCHEDS];     /* scheduled events */

extern char tag;                        /* handy local copy of the tag */

extern int event;                       /* handy local copy of the current event */

extern struct _nmap nmap[MAXNODES];     /* working node table */

extern int nn;                          /* current node, */

extern int nodetotal;                   /* number of nodes in the table */

extern struct _pkthdr pkthdr;           /* FidoNet packet header */

extern struct _mail mail;               /* MAIL.SYS */

extern struct _node node;               /* in memory node descriptor */

extern struct _sys sys;

extern char *_txbuf;            /* message display buff, ASCII upload */

extern int sysfiles;            /* total system files, */

extern int overwrite;           /* 1 == allow overwriting files */

extern int xferdisp;            /* 1 == display up/dn load status */

extern int rate;                /* baud rate */

extern int cd_flag;

extern int limit;

/* Carrier detected. Determine the baud rate, display a message to

tell humans what were doing, then get in sync with the sender. Once

in sync, receive the mail packet. */

rcv_mail() {

Fidonet Electronic Mail Protocol 20 Oct 84        Page 19

int i,n;

char pktname[80];

int msg_pkts,msg_blocks;

gtod(pktname);

lprintf("Call in @ %s\r\n",pktname);    /* log the call, */

cprintf(" * Incoming Call\r\n");

clr_clk();                      /* reset the clock, */

limit= 1;                       /* limit to connect, */

cd_flag= 0;                     /* watch Carrier Detect */

/* We get an abort if: timeout, no sync (human caller), actual file transfer

error, or packet received and sender hangs up. Totl_files is 0 if no

packets transferred. */

totl_blocks= 0;

totl_files= 0;

msg_pkts= 0;

msg_blocks= 0;

set_abort(0);                   /* trap carrier loss here, */

if (was_abort()) {

if (msg_pkts == 0) {

cprintf("          Receiving Mail Packet Aborted\r\n");

lprintf("\r\n*Aborted\r\n");

} else {

cprintf(" * Received %u packets\r\n",msg_pkts);

lprintf(" %u pkts %u blks OK\r\n",msg_pkts,msg_blocks);

if (totl_files > 0) {

cprintf("          Received %u files\r\n",totl_files);

lprintf("Recv'd Files\r\n");

}

}

cd_flag= 0;                     /* real CD, */

if (dsr()) discon();            /* hang up, delay for telco */

else delay(200);                /* delay for telco */

return(msg_pkts);

}

/* The sender whacks CR until it gets a CR back. It then waits for the

message to stop, sends a TSYNC, and starts the transfer. */

/* -------------------- One Minute to Complete This ---------------------- */

cprintf(" * Determining Baud Rate\r\n");

connect();                      /* determine baud rate, */

modout(CR);                     /* tell Tx we got baud right */

mprintf("\r\r\r\r\n   FidoNet Version %s\r\n",ver);

mprintf("   FidoNet Node #%u\r\n",mail.node);

mprintf("   Processing mail only.\r\n");

mcrlf();

cprintf(" * Waiting for Sync\r\n");

while (modin(100) != TSYNC)

Fidonet Electronic Mail Protocol 20 Oct 84        Page 20

limitchk();

/* ------------------- End One Minute to Complete -------------------- */

/* All connected. Transfer the mail packet. No time limit, but the

file receive will abort if there is an error. */

limit= 0;                       /* no time limit */

sprintf(pktname,"%u.in",pktnum++);      /* pick a new name, */

cprintf(" * Waiting for Mail Packet (%s)\r\n",pktname);

lprintf(" %s ",pktname);

crcmode= 1;                     /* CRC, */

filemode= XMODEM;               /* file xfer mode, */

recieve(pktname);               /* receive a packet, */

msg_pkts= totl_files;

msg_blocks= totl_blocks;        /* number of mail packets, */

totl_files= 0;

totl_blocks= 0;

/* Let extra EOTs, etc settle out, then flush it so the file receive

doesnt get the extra EOTs. */

delay(200);                     /* delay for last char */

while (modin(10) != TIMEOUT);

/* Wait for any incoming files. If there areent any, then the caller

will just hang up if an older version, or will send EOT if no files. */

filemode= TELINK;

recieve(mail.filepath);         /* get any files, */

delay(200);

frc_abort();                    /* error trap exit */

}

/* Send a mail packet to the remote. Dial the number, if error, return 0 to

indicate no connection. Also, 'minutes' upon return reflects the actual

connection time. */

send_mail() {

char pktname[80],fllname[80];

int n,f;

int msg_pkts,msg_blocks;

int online;

cprintf(" * Calling Fido%u %s %s\r\n",node.number,node.name,node.phone);

online= 0;                      /* no connect yet */

*_txbuf= '\0';                  /* in case no file, */

sprintf(fllname,"%u.fll",node.number);

f= _xopen(fllname,0);           /* read in file list, */

n= _xread(f,_txbuf,_TXSIZE);

_txbuf[n]= '\0';                /* null terminate it, */

_xclose(f);

Fidonet Electronic Mail Protocol 20 Oct 84        Page 21

gtod(pktname);

lprintf("Call out Fido%u @ %s\r\n",node.number,pktname);

sprintf(pktname,"%u.out",node.number);  /* local packet to send */

++nmap[nn].tries;               /* log another try, */

/* We get an abort if (1) it times out dialing, (2) we fail to connect

or (3) transmission sucessful, and receiver hung up. totl_files tells

us what happened. */

totl_blocks= 0;

totl_files= 0;

msg_pkts= 0;

msg_blocks= 0;

set_abort(0);

if (was_abort()) {

if (online) {

nmap[nn].time+= (minutes * 60) + seconds;

++nmap[nn].connects;

}

if (msg_pkts == 0) {

cprintf("          Packet Transmission Aborted\r\n");

lprintf("*Nothing sent\r\n");

} else {

cprintf(" * Sent %u packets %u files\r\n",

msg_pkts,totl_files);

lprintf(" Sent %u pkts %u files\r\n",msg_pkts,totl_files);

nmap[nn].success= 1;

_xdelete(pktname);      /* done with that packet */

_xdelete(fllname);

}

cd_flag= 0;

if (dsr()) discon();

return(msg_pkts);

}

/* Set the baud rate from the node list, and dial the number. Dialing is

aborted if either a no-connect from the modem, or a timeout if no modem

is connected. */

rate= 0;                                /* assume 300 baud, */

if (node.rate >= 1200) rate= 1;         /* unless higher */

baud(rate);

cd_flag= 1;                             /* dont bomb on no carrier */

/* -------------------- One Minute to Dial ---------------------- */

limit= 1;                               /* 1 min to dial, */

clr_clk();                              /* reset the clock, */

if (dial(node.phone) == 0) {

cprintf(" * No connection\r\n");

lprintf("*No connection\r\n");

Fidonet Electronic Mail Protocol 20 Oct 84        Page 22

frc_abort();                    /* do abort handler */

}

rate= 0;                                /* again for stupid hardware */

if (node.rate >= 1200) rate= 1;         /* that dials at fixed rates */

baud(rate);

cd_flag= 0;                             /* now watch carrier */

cprintf(" * Connected at %u baud\r\n",node.rate);

lprintf(" Connect @ %u\r\n",node.rate);

/* -------------------- One Minute to Connect and Sync --------------- */

/* Send CRs until we get a CR back, saying we detected baud correctly. */

online= 1;                              /* connected now */

clr_clk();                              /* reset clock, */

limit= 1;                               /* 1 min to connect, etc */

/*300*/ while (modin(100) != TIMEOUT)           /* quiet line, */

limitchk();

for (n= 8; --n;) {                      /* limited tries for baud test */

modout(CR);                     /* send CR CR until we */

/**/            delay(20);                      /* get one back, */

/**/            modout(CR);

if (modin(100) == CR) break;

}

if (n == 0) {

lprintf("*autobaud failed\r\n");

frc_abort();                    /* give up if no response */

}

/* We got a CR from the receiver. Baud rate detected. Now, wait for the

prompt junk to go away, then send our sync character to say we're ready

to send. */

/*100*/ while (modin(50) != TIMEOUT)            /* wait for quiet line, */

limitchk();

modout(TSYNC);                          /* send our sync, */

lprintf(" Sent TSYNC\r\n");

/* -------------------- End One Minute Connect and Sync -------------- */

limit= 0;                               /* no limit now */

cprintf(" * Sending Packet\r\n");

filemode= XMODEM;

crcmode= 1;

transmit(pktname);

msg_pkts= totl_files;

msg_blocks= totl_blocks;

totl_files= 0;

totl_blocks= 0;

if (strlen(_txbuf) == 0) {              /* if no files, just */

Fidonet Electronic Mail Protocol 20 Oct 84        Page 23

modout(EOT);                    /* send EOT, */

} else {

filemode= TELINK;               /* now send any files, */

transmit(_txbuf);

}

delay(500);

frc_abort();                            /* do termination code */

}